Digital Speech Processing and Noise Reduction

10.1 Implementation of Spectral Subtraction for Speech Noise Reduction

Program (Matlab Code):

clc;

clear;

close all;

%% Sampling Parameters

Fs = 8000;                 % Sampling frequency

t = 0:1/Fs:3;              % Time vector (3 seconds)

%% Clean Speech Signal (Example)

x = sin(2*pi*200*t) + 0.5*sin(2*pi*400*t);

%% Add Gaussian Noise

noise = 0.3 * randn(size(x));

xn = x + noise;

%% FFT of Noisy Signal and Noise

Y = fft(xn);

N = fft(noise);

%% Spectral Subtraction

magY = abs(Y);

magN = abs(N);

phaseY = angle(Y);

magX = max(magY - magN, 0);    % Prevent negative values

Xe = magX .* exp(1j * phaseY);

%% Enhanced Signal

xe = real(ifft(Xe));

%% ---------------- Time Domain Signals ----------------

figure;

subplot(3,1,1);

plot(t,x,'b');

title('Clean Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(3,1,2);

plot(t,xn,'r');

title('Noisy Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(3,1,3);

plot(t,xe,'g');

title('Enhanced Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

%% ---------------- Spectrograms ----------------

figure;

subplot(3,1,1);

spectrogram(x,256,200,256,Fs,'yaxis');

title('Clean Signal Spectrogram');

subplot(3,1,2);

spectrogram(xn,256,200,256,Fs,'yaxis');

title('Noisy Signal Spectrogram');

subplot(3,1,3);

spectrogram(xe,256,200,256,Fs,'yaxis');

title('Enhanced Signal Spectrogram');

%% ---------------- Magnitude Spectrum ----------------

Nfft = length(x);

f = (0:Nfft-1) * Fs / Nfft;

figure;

subplot(3,1,1);

plot(f,abs(fft(x)));

title('Clean Signal Spectrum');

xlabel('Frequency (Hz)');

ylabel('Magnitude');

xlim([0 Fs/2]);

grid on;

subplot(3,1,2);

plot(f,abs(fft(xn)));

title('Noisy Signal Spectrum');

xlabel('Frequency (Hz)');

ylabel('Magnitude');

xlim([0 Fs/2]);

grid on;

subplot(3,1,3);

plot(f,abs(fft(xe)));

title('Enhanced Signal Spectrum');

xlabel('Frequency (Hz)');

ylabel('Magnitude');

xlim([0 Fs/2]);

grid on;

%% ---------------- SNR Calculation ----------------

snr_before = 10 * log10(sum(x.^2) / sum((xn - x).^2));

snr_after = 10 * log10(sum(x.^2) / sum((xe - x).^2));

fprintf('SNR Before Enhancement = %.2f dB\n', snr_before);

fprintf('SNR After Enhancement  = %.2f dB\n', snr_after);Output