Design of High Pass Filter using Windowing Techniques

5.2. Generate the coefficients of an FIR High-Pass filter using Hamming windowing techniques

Program (Matlab Code):

clc; clear; close all;

%% --- Input Parameters ---

fp = 1000; % High-pass cutoff frequency (Hz)

fs = 8000; % Sampling frequency (Hz)

N = 50; % Filter order

wc = fp/(fs/2); % Normalized cutoff (0–1)

%% --- FIR High Pass using Hamming Window ---

b_hamm = fir1(N, wc, 'high', hamming(N+1));

disp('--- High-Pass Coefficients (Hamming Window) ---');

disp(b_hamm');

%% --- Frequency Response Plots ---

figure;

freqz(b_hamm,1);

title('High-pass FIR using Hamming Window');

title('High-pass FIR using Hanning Window');