Finite Word length Effects

6.1 Uniform Quantization and Quantization Error Analysis of a Discrete-Time

Sinusoidal Signal Using MATLAB

Program (Matlab Code):

clc;

clear;

close all;

%% Signal

fs = 1000;

f = 50;

t = 0:1/fs:0.1;

x = sin(2*pi*f*t);

%% Quantization Levels

L = [8 16 32];

colors = {'r','g','m'};

figure;

for i = 1:length(L)

% Quantization

xq{i} = round(((x + 1) * (L(i) - 1) / 2)) * (2 / (L(i) - 1)) - 1;

% Error & MSE

e{i} = x - xq{i};

mse(i) = mean(e{i}.^2);

% Plot Quantized Signals

subplot(4,1,i+1);

stairs(t, xq{i}, colors{i}, 'LineWidth', 1.2);

title(['Quantized Signal (' num2str(L(i)) ' Levels)']);

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

end

%% Original Signal

subplot(4,1,1);

plot(t, x, 'b', 'LineWidth', 1.5);

title('Original Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

%% Display MSE

fprintf('MSE for 8 Levels  = %f\n', mse(1));

fprintf('MSE for 16 Levels = %f\n', mse(2));

fprintf('MSE for 32 Levels = %f\n', mse(3));

%% Error Plots

figure;

for i = 1:3

subplot(3,1,i);

plot(t, e{i}, 'LineWidth', 1.2);

title(['Quantization Error (' num2str(L(i)) ' Levels)']);

xlabel('Time (s)');

ylabel('Error');

grid on;

end