Finite Word length Effects

6.2 Analysis of Finite Word Length Effects in Digital Signal Representation Using

Truncation and Rounding Techniques in MATLAB

Program (Matlab Code):

clc; clear; close all

n=0:79;

x=sin(2*pi*n/40);

q=8;

xt=floor(x*q)/q;

xr=round(x*q)/q;

et=x-xt;

er=x-xr;

% Fig 1

figure

plot(n,x,'b',n,xr,'r','LineWidth',1.5)

grid on

title('Original vs Quantized')

legend('Original','Quantized')

% Fig 2

figure

plot(n,x,'b',n,xt,'r',n,xr,'g','LineWidth',1.5)

grid on

title('Truncation vs Rounding')

legend('Original','Truncated','Rounded')

% Fig 3

figure

stem(n,et,'r')

hold on

stem(n,er,'g')

grid on

title('Quantization Error')

legend('Truncation','Rounding')

% Fig 4

figure

bar([mean(et.^2) mean(er.^2)])

set(gca,'XTickLabel',{'Truncation','Rounding'})

grid on

title('MSE Comparison')

ylabel('MSE')